References

[1]  S. Santos, P. Costa, and A. Rocha, "IT/OT convergence in Industry 4.0: risks and analysis of the problems," in Proc. 18th Iberian Conf. on Information Systems and Technologies (CISTI), 2023.

[2]  L. D. Xu, E. L. Xu, and L. Li, "Industry 4.0: state of the art and future trends," Int. J. Prod. Res., vol. 56, no. 8, pp. 2941-2962, 2018.

[3]  Enterprise-Control System Integration - Part 1: Models and Terminology, ISA-95.00.01, 2010.

[4]  Modbus Organization, Modbus Application Protocol Specification V1.1b3, 2012.

[5]  Information technology - Message Queuing Telemetry Transport (MQTT) v3.1.1, ISO/IEC 20922, 2016.

[6]  InfluxData, "Telegraf 1.28 documentation," 2024. [Online]. Available: https://docs.influxdata.com/telegraf/. (Accessed: Jun. 2026).

[7]  InfluxData, "InfluxDB 2.7 documentation," 2024. [Online]. Available: https://docs.influxdata.com/influxdb/v2/. (Accessed: Jun. 2026).

[8]  Grafana Labs, "Grafana documentation," 2024. [Online]. Available: https://grafana.com/docs/grafana/latest/. (Accessed: Jun. 2026).

Author Biography

Mr. Patthanan Bhandhumanee, King Mongkut’s University of Technology North Bangkok, Department of Electrical and Computer Engineering. [birthdate, hometown, education, contact]

Appendix A  ESP32-S3 Firmware Design Overview

This appendix summarises the design of the ESP32-S3 gateway firmware at the level of modules and responsibilities; full source code is omitted by design and lives in the project repository. The firmware is an Arduino-core application built with PlatformIO. It is organised as a single cooperative, non-blocking loop that runs on a two-second cycle and never calls delay(), so that no role can starve another.

A.1  Concurrent Roles

The loop services four roles in turn, each implemented as a small state object polled once per pass:

•  Modbus RTU master - polls the XY-MD02 (FC04) and the SDM230 power meter (FC03/FC04) over RS-485, frames each request with a CRC-16, recognises exception responses (function code with the high bit set), and records a unified per-slave status code rather than throwing on error.

•  MQTT publisher - serialises the latest readings to a compact JSON document and publishes at QoS 1. On a device-side fault the affected value fields are omitted and only the status code is sent, so a downstream consumer never ingests a fabricated number.

•  Modbus TCP slave - serves the same readings on TCP port 502 as holding registers HR0-HR20, so any IP host (including the PLC acting as master) can read them without touching the serial bus.

•  Watchdog - a software timer that flags a stalled cycle and forces a clean restart, guarding against a wedged transceiver or broker socket.

A.2  Holding-Register Map (HR0-HR20)

The TCP slave exposes a fixed register map. Scaled integers avoid floating-point on the wire for the simple sensor; the SDM230's IEEE-754 floats are published as high/low word pairs.

| Register | Contents | Encoding |
| --- | --- | --- |
| HR0 | Temperature | signed int, x100 degC |
| HR1 | Relative humidity | unsigned int, x100 %RH |
| HR2 | Per-slave status code | enum (OK / CRC / timeout / exception) |
| HR3 | Poll counter | unsigned int, wraps |
| HR4-HR19 | SDM230 measurements | 8 x IEEE-754 float, hi/lo word pairs |
| HR20 | Gateway uptime | seconds |

A.3  Build and Resource Budget

Built with the Arduino core over FreeRTOS on PlatformIO, the production firmware occupies 946 KB of the 1310 KB application partition (72.2 percent) and 48.5 KB of 327.6 KB RAM (14.8 percent), confirming that a complete dual-protocol gateway fits a low-cost MCU with ample headroom for future features such as an over-the-air update path or a telemetry ring buffer.

Appendix B  Docker Compose and Telegraf Configuration

This appendix documents the containerised IT layer on the Raspberry Pi 4 at the level of services and intent; the full compose.yml and telegraf.conf are omitted and kept in the repository. Every component is an open-source Docker image, declared in one Docker Compose file with pinned tags and secrets injected through environment variables, so the entire stack is reproducible by another student with a single command.

B.1  Service Roles

•  Mosquitto 2.0 (MQTT broker) - terminates publishes from the gateway on TCP 1883 and bridges to browsers on 9001 (WebSockets); Docker log rotation is enabled to bound disk use.

•  InfluxDB 2.7 (time-series database) - stores telemetry in the bucket 'sensors' under the organisation 'weatherstation'; queried with Flux.

•  Telegraf 1.28 (collection agent) - its mqtt_consumer input subscribes at QoS 1 and parses the JSON, its influxdb_v2 output writes to the database, and additional inputs gather Raspberry Pi host metrics (CPU, memory, disk, temperature) and network reachability.

•  Grafana (dashboards) - renders the weather, power, host, and network panels, with an anonymous read-only viewer role for an unattended kiosk display.

B.2  Running Services - Port Reference

Network-accessible services as deployed on the Raspberry Pi 4. (Remote-access tunnelling was evaluated but is excluded from the on-premises architecture described in this thesis.)

| Port | Service | Type | Image / process |
| --- | --- | --- | --- |
| TCP 1883 | Mosquitto MQTT | Docker | eclipse-mosquitto:2.0 |
| TCP 9001 | Mosquitto WebSocket | Docker | WebSocket bridge |
| TCP 8086 | InfluxDB | Docker | influxdb:2.7 |
| UDP/TCP 8092/8094 | Telegraf inputs | Docker | telegraf:1.28 |
| TCP 3000 | Grafana | Docker | grafana/grafana |
| TCP 1880 | Node-RED (prototyping) | Docker | nodered/node-red |
| TCP 22 | SSH | System | host management |

Appendix C  Modbus Library Evaluation Scorecard

This appendix records the full evaluation scorecard behind the selection in Section 4.1. Eight open-source Modbus gateway libraries were evaluated on identical hardware. Beyond the latency, flash, RAM, and stress results of Table 4.1, each library was rated on ten qualitative criteria.

C.1  Qualitative Criteria Matrix

Ratings: Full = first-class support; Part = partial or workaround; None = unsupported. Maintenance is as of the evaluation date.

| Library | Framework | Maint. | RTU+TCP | Docs | Footprint |
| --- | --- | --- | --- | --- | --- |
| emelianov/modbus-esp8266 | Arduino | Full | Full | Full | Small (rec.) |
| eModbus/eModbus | Arduino | Part | Full | Full | Medium |
| espressif/esp-modbus | ESP-IDF | Full | Full | Full | Small |
| maxx-ukoo/esp32-modbus-tcp2rtu | ESP-IDF | None | Full | Part | Small |
| tobiasfaust/SolaxModbusGateway | Arduino | Part | Full | Part | Large |
| vwetter/esp32-modbus-gateway | Arduino | Part | Full | Part | Medium |
| harihanv/esp32-modbus-gateway | Arduino | None | Full | Part | Medium |
| zivillian/esp32-modbus-gateway | Arduino | Part | Full | Part | Large |

C.2  Selection Rationale

Latency was not the deciding criterion: every candidate sits within about 10 ms of the ~24 ms round-trip wire-time floor, far below the platform's two-second telemetry cycle. The choice therefore weighted maintenance, the three roles the platform needs (RTU master, Modbus TCP slave, multi-client TCP), footprint, and stress robustness. emelianov/modbus-esp8266 wins on the combination of active maintenance, full role coverage, the smallest binary among the TCP-capable libraries, and a clean pass of the stress suite.

Appendix D  IoT-Sniffer Tool and Verification Use

IoT-Sniffer is a passive, real-time protocol analyser written in Python and used in this project as a wire-level verification instrument for the gateway and the benchmark. Unlike a general-purpose tool it needs no tap or port-mirror: it runs scapy's AsyncSniffer directly on the host NIC behind a kernel BPF pre-filter and decodes Modbus TCP (port 502) and MQTT (port 1883, and over WebSockets after stripping the RFC 6455 framing) at the application layer.

D.1  Architecture

A capture thread dispatches each packet by its (src, dst) flow tuple to a TCP reassembler that handles out-of-order segments; reassembled bytes feed pure-Python parsers. The Modbus parser decodes the seven-byte MBAP header (transaction id, protocol id, length, unit id) and the function code, flagging exception responses when the high bit of the function code is set. The MQTT parser decodes the fixed-header type and flags nibbles and the variable-length remaining-length field, covering CONNECT, PUBLISH, PUBACK, SUBSCRIBE, and the keep-alive packets. Frames are persisted to SQLite in write-ahead-log mode and pushed to a browser dashboard over WebSocket, alongside a one-hertz metrics snapshot.

D.2  Role in This Project

The tool was used to confirm that the bytes on the wire matched the firmware's intent: that MBAP fields and unit ids were correct, that the SDM230 IEEE-754 floats were transmitted in the expected word order, and that each FC03 request from a client was paired with exactly one response from the gateway. Reading the decoded request/response pairing on the wire provided an independent check on the host-side timing used in the library benchmark of Section 4.1, and helped diagnose framing and byte-order issues during development before they could corrupt the stored telemetry.

D.3  Dashboard Views

•  Packet inspector - a protocol-coloured frame rail; clicking a frame expands a field map showing each MBAP and PDU byte range with its decoded value.

•  Sequence view - a UML-style message-sequence chart with device lifelines and time flowing downward, used to confirm request/response pairing.

•  Topology and matrix views - device-to-device graphs with edge weight scaled by packet count, used to see which flows dominate a capture.

•  Performance tab - p50/p95/p99 latency, inter-arrival jitter, and frame-size distributions per protocol, with one-click CSV export for offline analysis.

Appendix E  Hardware Reference

This appendix lists the key electrical, mechanical, and communication specifications of the platform's hardware. Figures referenced here appear in the List of Figures.

E.1  Waveshare ESP32-S3-Relay-6CH (MCU)

A compact industrial relay board built around the Espressif ESP32-S3-WROOM-1 module, chosen as the edge gateway for its on-board RS-485 transceiver, six relays, PSRAM, and PlatformIO support.

•  SoC: ESP32-S3 (Xtensa LX7 dual-core, up to 240 MHz), Wi-Fi 802.11 b/g/n + BLE 5

•  Memory: 16 MB Quad-SPI flash + 8 MB PSRAM, 512 KB on-chip SRAM

•  Relays: 6 x SPDT, 250 VAC/10 A or 30 VDC/10 A, GPIO-driven through optocoupler isolation

•  Field bus: on-board RS-485 (half-duplex) transceiver on a hardware UART

•  Supply: 7-36 V DC via screw terminal, or 5 V via USB-C (also used for flashing/debug); logic 3.3 V

•  Power draw: ~0.6-1.7 W depending on relay load

•  Dimensions: 65 x 56 mm; toolchain: Arduino core / ESP-IDF via PlatformIO

E.2  Siemens LOGO! 24CE - firmware V8.4 (PLC)

A compact 24 V DC micro-PLC with a native Modbus TCP server, programmed only through the proprietary LOGO! Soft Comfort in IEC 61131-3 ladder or function-block diagrams.

•  Part number: 6ED1052-1CC08-0BA2; supply 24 V DC; power consumption <= 4.5 W

•  I/O: 8 x 24 V DC digital inputs (I1-I4 usable as 0-10 V analogue), 4 x transistor outputs 24 V DC / 0.3 A

•  Network: 1 x 10/100 Mbit/s Ethernet; Modbus TCP server and client on port 502, up to 8 connections

•  Modbus functions: FC01/02/03/05/06/15/16; V-memory word-mapped to holding registers, coils mapped to Q1-Q4

•  Memory: 400 function blocks, 4000-byte retentive V-memory; battery-backed real-time clock

•  Certifications: CE, UL, cUL, FM, ATEX Zone 2; DIN-rail mount; 72 x 90 x 55 mm

E.3  Raspberry Pi 4 Model B, 2 GB (SBC)

The IT-layer host running the TIG stack and Mosquitto under Docker Compose, driving a Grafana kiosk display.

•  SoC: Broadcom BCM2711 (quad-core Cortex-A72, 1.5 GHz); RAM: 2 GB LPDDR4

•  Network: Gigabit Ethernet; OS: Raspberry Pi OS Lite 64-bit (Debian Bookworm)

•  Supply: 5.1 V / 3 A via USB-C; idle ~3.4 W, peak ~7.6 W under display + Docker load

•  Runtime: Docker Engine 24.x + Compose v2; 85 x 56 mm

E.4  XY-MD02 Temperature / Humidity Transmitter (field instrument)

A DIN-rail temperature and humidity transmitter based on the Sensirion SHT20, a pure Modbus RTU field device with no IP stack.

•  Sensing: SHT20; temperature +/-0.3 degC, humidity +/-3 %RH

•  Protocol: Modbus RTU over RS-485, 9600 baud 8-N-1, slave address 0x01

•  Registers (FC04): temperature x10 and humidity x10 as 16-bit integers

•  Supply: 10-30 V DC, < 30 mA

E.5  Eastron SDM230-Modbus Single-Phase Power Meter (field instrument)

A DIN-rail single-phase digital energy meter (Eastron SDM230-Modbus), direct-connected up to 100 A, in a two-module DIN-rail form factor with a backlit LCD, providing real industrial measurement semantics and exposing values as IEEE-754 float register pairs.

•  Measurements (15 parameters): voltage, current, frequency, power factor, active/apparent/reactive power, and imported/exported/total active and reactive energy

•  Encoding: 32-bit IEEE-754 floats across paired input registers (big-endian word order)

•  Protocol: Modbus RTU over RS-485 (2400/4800/9600 baud); reads issued by the ESP32-S3 with FC03/FC04; configurable pulsed energy output

•  Rating: single-phase 230 V, direct connection up to 100 A; MID Class B certified (EN50470-3); two-module DIN-rail mount

Appendix F  IIoT Platform Comparison Matrix

This appendix places the platform's three controller classes side by side across six engineering dimensions, expanding the summary of Table 5.1. Alternative microcontrollers surveyed but not adopted (STM32 F7-series, Arduino Mega) are noted in the discussion; the matrix below is restricted to the devices actually used so every rating is grounded in this build. Ratings: Full = first-class native support, Part = partial or add-on, None = unsupported.

F.1  Comparison Matrix (MCU vs SBC vs PLC)

| Dimension | ESP32-S3 (MCU) | RPi 4 (SBC) | LOGO! 24CE (PLC) |
| --- | --- | --- | --- |
| Modbus RTU (RS-485) | Full (on-board) | Part (USB adapter) | Full (native) |
| Modbus TCP | Full (Wi-Fi/IP) | Full (Linux stack) | Full (native server) |
| MQTT | Full (native client) | Full (broker host) | None |
| Determinism | Soft real-time | Non-deterministic | Hard real-time |
| Field I/O | GPIO/ADC/UART/RS-485 | GPIO; no analogue | 24 V DI, 0-10 V AI, relays |
| Edge compute | Constrained (KB RAM) | Full (Linux/Docker) | Fixed scan logic |
| Toolchain | Open (Arduino/PIO) | Open (Linux/Docker) | Proprietary (Soft Comfort) |
| Certification | None | None | CE/UL/FM/ATEX |
| Indicative cost | ~1,895 THB | ~1,900-3,200 THB | ~5,000-6,500 THB |

F.2  Reading the Matrix

No single device dominates: the MCU leads on cost-per-channel, native field interfaces, and an open toolchain but lacks an operating system and certification; the SBC leads on edge compute and software richness but cannot guarantee hard real-time; the PLC alone offers certified, bounded-scan control of mains-level actuators but is the most expensive and the most closed. The matrix is therefore the evidence behind the thesis claim that the three classes are complementary layers, not competing alternatives.