# Chapter 1  Introduction

## 1.1  Background and Motivation

Industry 4.0 has pushed factories to expose shop-floor data to information-technology (IT) systems for monitoring, analytics, and remote supervision. Yet the operational-technology (OT) equipment that runs physical processes still speaks decades-old protocols. Modbus, introduced in 1979, remains one of the most widely deployed field protocols, and in its serial RTU form it runs over RS-485 with no native path to an IP network [4]. Replacing such field hardware wholesale is rarely economic, so the practical problem is not replacement but integration: how to lift legacy OT signals into modern IT dashboards at low cost and without compromising the determinism the OT side depends on.

Three classes of controller dominate this landscape, and a student entering the field must understand all three: the microcontroller (MCU), the single-board computer (SBC), and the programmable logic controller (PLC). They differ not only in compute and I/O but in something this work treats as a first-class concern — their software ecosystems. The MCU and SBC are programmed with well-known open-source software (the Arduino core and PlatformIO, Linux and Docker, the Telegraf-InfluxDB-Grafana “TIG” stack, Node-RED, and the MQTT broker Mosquitto), whereas the PLC is tied to a proprietary vendor toolchain — for the Siemens LOGO! used here, LOGO! Soft Comfort. That contrast in openness is itself a lesson about vendor lock-in in OT.

## 1.2  Problem Statement

Teaching IT/OT integration well requires hardware a student can touch, break, and instrument. Commercial IIoT trainers are expensive and closed, hiding exactly the wire-level behaviour a learner needs to see. Two gaps follow. First, no low-cost bench exposes all three controller classes and real industrial protocols together. Second, although many open-source ESP32 Modbus gateway libraries exist, there is no empirical comparison guiding which to use, so a builder chooses blind.

## 1.3  Objectives

1)  Design and build a low-cost educational IIoT laboratory platform that integrates the three main hardware classes of modern automation — a single-board computer (SBC, Raspberry Pi 4), a microcontroller (MCU, ESP32-S3), and a programmable logic controller (PLC, Siemens LOGO! 24CE).

2)  Demonstrate integration between IT and OT devices with an emphasis on an open-source software stack, adopting the tools most widely used in IIoT applications — the MQTT broker (Mosquitto), Node-RED, and the TIG stack (Telegraf, InfluxDB, Grafana) — as the platform's edge and IT layers.

3)  Implement a Modbus RTU-to-TCP/MQTT gateway on the MCU and evaluate open-source ESP32-S3 Modbus gateway libraries, benchmarking the gateway-capable candidates on real hardware to select one, and measuring the resulting gateway's latency, throughput, and flash/RAM footprint.

4)  Develop an IoT-Sniffer that decodes Modbus TCP and MQTT to observe network activity on the wire and measure the network performance of the platform's IoT devices.

## 1.4  Scope and Limitations

The platform is a single-site LAN bench. In scope: Modbus RTU and TCP, MQTT, the TIG stack, and the passive sniffer. Out of scope: OPC-UA, industrial certification, hazardous-area (ATEX) deployment, and production hardening. Library results are obtained on one ESP32-S3 hardware revision and may shift on other revisions.

## 1.5  Thesis Organization

Chapter 2 reviews IT/OT convergence, the common hardware and software found in IT and OT, the Modbus and MQTT protocols, the three controller classes and their open-source versus proprietary toolchains, the dashboard stack, and related work. Chapter 3 presents the platform architecture and the implementation of each layer, including the MCU gateway firmware, the IT stack, and the sniffer. Chapter 4 reports the evaluation: the Modbus library benchmark and gateway performance. Chapter 5 discusses the findings, the fit of each controller class, limitations, and future work.