# Chapter 5  Discussion and Conclusion

This chapter summarises the system that was built, discusses the problems and obstacles encountered during the work together with their solutions, and offers recommendations for further development.

## 5.1  System Overview

The platform integrates the three controller classes on a single local-area network. The ESP32-S3 microcontroller acts as the edge gateway, converting Modbus RTU to Modbus TCP and MQTT; the Raspberry Pi 4 single-board computer hosts the IT layer, running the MQTT broker and the TIG stack in containers; and the Siemens LOGO! 24CE PLC provides deterministic control. Data from the field instruments flows through the edge gateway up to the Grafana dashboards in real time. The results confirm that a single low-cost microcontroller can bridge legacy serial OT equipment to a modern IT dashboard within its firmware budget. Table 5.1 summarises the three controller classes side by side, showing that each device suits a different, complementary role rather than competing with the others.

Table 5.1  Comparison of the three controller classes used in the platform.

| Attribute | MCU — ESP32-S3 | SBC — Raspberry Pi 4 | PLC — LOGO! 24CE |
| --- | --- | --- | --- |
| Operating system | None (cooperative loop) | Full Linux | Proprietary firmware |
| Determinism | Soft real-time | Non-deterministic | Hard real-time |
| Native I/O | GPIO/ADC/UART/RS-485 | GPIO + USB; no analog | 24 V DI, 0-10 V AI, relays |
| Networking | Wi-Fi / BLE | Gigabit Ethernet | 10/100 Modbus TCP |
| Supply voltage | 7-36 V DC (3.3 V logic) | 5 V DC | 24 V DC |
| Power draw | ~0.6-1.7 W | ~3.4-7.6 W | <=4.5 W at 24 V DC |
| Toolchain | Open (Arduino/PlatformIO) | Open (Linux/Docker) | Proprietary (Soft Comfort) |
| Certification | None | None | CE, UL, FM, ATEX |
| Approx. unit cost | ~1,895 THB | ~1,900-3,200 THB | ~5,000-6,500 THB |

The MCU delivers the lowest cost-per-channel and lowest power and is the correct field-edge gateway, but it lacks an operating system and certification. The SBC offers unmatched software richness — any Linux package or container runs unmodified — but its preemptive scheduler cannot guarantee hard real-time behaviour, so it belongs in the data plane, not in control. The PLC is the only device that drives mains-level actuators under a bounded, certified scan cycle, which is a safety property rather than a mere performance figure, but it is the most expensive and the most closed. Substituting one class for another across layers trades exactly these properties. The assignment of each device to its role is therefore an engineering consequence, not a preference.

## 5.2  Problems, Obstacles, and Solutions

Several problems and obstacles arose during design and development, together with their solutions. First, the open-source Modbus gateway libraries for the ESP32 are numerous and vary widely in quality, in both maintenance and feature coverage, which made selection difficult; this was addressed by defining explicit evaluation criteria and benchmarking on real hardware for a systematic selection, as reported in Chapter 4. Second, RS-485 communication is sensitive to wiring, A/B polarity, and bus termination, causing intermittent CRC errors; this was resolved by checking the wiring, sharing a common ground, and matching the baud rate on all devices. Third, the Raspberry Pi 4 used in this work is the 2 GB variant, yet it must host several containers at once (Mosquitto, InfluxDB, Telegraf, Grafana, and Node-RED), making memory the main constraint of the IT layer; this was mitigated by running the headless Raspberry Pi OS Lite, keeping only the necessary services, and monitoring memory usage on the host-status dashboard collected by Telegraf to confirm sufficient headroom during continuous operation. Fourth, Node-RED was initially deployed as a stand-alone container outside the Docker Compose stack, and its flows could not open Modbus TCP connections to the gateway because the separately deployed container's network configuration did not match the rest of the services; moving Node-RED into the same Docker Compose file as the other services, so that all services share one network and are managed with a single command, restored normal Modbus TCP operation.

## 5.3  Recommendations

This work can be extended in several directions. On the edge gateway, a telemetry ring buffer should be added to tolerate Wi-Fi outages, and an over-the-air (OTA) firmware update mechanism should be added to ease maintenance. On the IT layer, scheduled InfluxDB backups, container health checks, and Grafana threshold alerting should be added to strengthen the reliability of the data plane. On the communication side, adding the OPC-UA protocol should be studied to broaden interoperability with other industrial systems. Finally, on adoption, the platform should be trialled with students in a laboratory course to assess its educational value, and taken further toward real deployment or higher-level research.