# ESP32 Modbus Library Evaluation — Hardware Test Report

Patthanan Bhandhumanee · 2026-05-14

# 1. Goal

Evaluate the ten ESP32 Modbus projects listed in modbus_lib_eval_2026-05-11.pdf by flashing each candidate library onto an ESP32-S3 (device under test) and exercising its Modbus RTU master code against an ESP32-C6 mock slave over RS485. The evaluation measures whether each candidate builds on a current Arduino-ESP32 core or ESP-IDF toolchain, what flash and RAM it consumes, and whether it can successfully complete a fixed FC3 / FC6 / FC16 test cycle.

# 2. Hardware Setup

| Role | Board |
| --- | --- |
| RTU slave fixture | ESP32-C6 DevKitC-1 |
| DUT | ESP32-S3 4D Systems Gen4 R8N16 |

Boards are connected through an auto-direction RS485 transceiver (e.g. MAX13487). DE / RE pins are not driven by firmware. Common ground is shared and the A / B lines are wired straight through.

| Board | Signal | GPIO |
| --- | --- | --- |
| ESP32-S3 DUT | TX to RS485 DI | 9 |
| ESP32-S3 DUT | RX from RS485 RO | 8 |
| ESP32-C6 fixture | TX to RS485 DI | 4 |
| ESP32-C6 fixture | RX from RS485 RO | 5 |

# 3. Methodology

Each PlatformIO environment builds a tiny Arduino sketch or ESP-IDF app_main that uses one candidate library to drive the same test cycle against the C6 mock slave. The sketch style is intentionally flat: globals plus setup() and loop() (or app_main with while(1)), free helper functions, no custom classes, no shared harness — the way an Arduino IDE user would start from a library example. Calls into a library's own classes are allowed; those are upstream code.

Each cycle of the loop runs four steps:

T1: FC3 read holding registers 0..4 — expect 100, 200, 300, 400, 500.

T2: FC6 write reg 0 = 4242.

T3: FC3 read reg 0 — expect 4242 (verifies T2).

T4: FC16 write regs 0..2 = 100, 200, 300 (restores initial state).

PASS / FAIL counters increment per step and a "CYCLE SUMMARY: n/m PASS" line is printed after every cycle. The C6 slave keeps holding registers 0..9 initialised to 100, 200, …, 1000 and supports the same three function codes plus standard Modbus exception responses for illegal address or quantity.

# 4. Candidate-by-Candidate Strategy

| Candidate | Framework | Library exercised | Strategy |
| --- | --- | --- | --- |
| Rob2011/ESP32.Modbus-TCP-gateway | Arduino | emelianov/modbus-esp8266 ModbusRTU | DUT sketch uses the sync ModbusRTU master API. |
| vwetter/esp32-modbus-gateway | Arduino | raw HardwareSerial | Vwetter's gateway hand-rolls RTU on serial; DUT mirrors that. |
| zivillian/esp32-modbus-gateway | Arduino | eModbus ModbusClientRTU | Upstream PIO project fails on Arduino core 3 (HTTP_GET / ESPAsyncWebServer clash). DUT exercises the eModbus layer it wraps. |
| harihanv/esp32-modbus-gateway | Arduino | raw HardwareSerial | ArduinoRS485 1.1.1 does not compile on ESP32 (expects SERIAL_PORT_HARDWARE / A5 / A6). DUT uses raw RTU, matching harihanv's own code path. |
| tobiasfaust/SolaxModbusGateway | Arduino | raw HardwareSerial | Solax gateway's modbus class is tied to LittleFS / MQTT / Solax registers. Not a reusable library. DUT mirrors their RS485 test sketch. |
| eModbus/eModbus | Arduino | eModbus ModbusClientRTU | Direct library use; modeled on examples/RTU04example. |
| NamNamIoT/ESP32 | Arduino | 4-20ma/ModbusMaster | Their Modbus_RTU.ino uses ModbusMaster; DUT mirrors it on Serial1. |
| maxx-ukoo/esp32-modbus-tcp2rtu | ESP-IDF | esp-modbus (their gateway wraps it) | Full TCP↔RTU gateway needs SPIFFS / MQTT / WiFi. DUT runs the same esp-modbus master path their modbus.c uses. |
| espressif/esp-modbus | ESP-IDF | esp-modbus v2 mbc_master_send_request | Minimal app_main with serial master API. |
| rosenrot00/esphome_modbus_bridge | ESPHome | — (excluded) | ESPHome is not a PlatformIO framework. Excluded; evaluate with `esphome compile` separately. |

# 5. Build Environment

PlatformIO Core 6.1.19 with the pioarduino platform-espressif32 fork (55.3.38) driving Arduino-ESP32 core and ESP-IDF 5.5.4. Arduino envs build with framework = arduino; the two espressif candidates build with framework = espidf. Both frameworks coexist in the same platformio.ini through separate [env:*] blocks.

Arduino source files are wrapped in #ifdef ARDUINO so the ESP-IDF builds compile them to empty translation units instead of failing to find <Arduino.h>. The two ESP-IDF main.c files use #ifdef DUT_ESP_MODBUS and #ifdef DUT_MAXX_UKOO so only one app_main is emitted per ESP-IDF env, avoiding link conflicts.

The pioarduino espidf builder injects an unquoted -fmacro-prefix-map=$PROJECT_DIR=. compiler flag. The project directory ends in "Senior Project/modbus_lib_eval/ESP32_c6_s3_modbus", and the unescaped space causes gcc to split the flag and fail. The workaround is to rsync the project to a no-space path (used in this experiment: /tmp/modbus_eval_real) and build the two ESP-IDF envs from there. The eight Arduino envs build directly from the original path.

# 6. Build & Resource Results

All ten environments compile successfully:

| Env | Framework | Status | Flash (B) | Flash total | Flash % | RAM (B) | RAM total | RAM % |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| esp32-c6-fixture | arduino | SUCCESS | 252,882 | 1,310,720 | 19.3% | 13,852 | 327,680 | 4.2% |
| dut_rob2011_arduino_s3 | arduino | SUCCESS | 310,437 | 3,145,728 | 9.9% | 22,156 | 327,680 | 6.8% |
| dut_vwetter_arduino_s3 | arduino | SUCCESS | 300,501 | 3,145,728 | 9.6% | 21,996 | 327,680 | 6.7% |
| dut_zivillian_arduino_s3 | arduino | SUCCESS | 384,105 | 3,145,728 | 12.2% | 22,932 | 327,680 | 7.0% |
| dut_harihanv_arduino_s3 | arduino | SUCCESS | 300,213 | 3,145,728 | 9.5% | 21,996 | 327,680 | 6.7% |
| dut_tobiasfaust_arduino_s3 | arduino | SUCCESS | 300,157 | 3,145,728 | 9.5% | 21,996 | 327,680 | 6.7% |
| dut_emodbus_arduino_s3 | arduino | SUCCESS | 384,033 | 3,145,728 | 12.2% | 22,940 | 327,680 | 7.0% |
| dut_namnam_iot_arduino_s3 | arduino | SUCCESS | 301,573 | 3,145,728 | 9.6% | 22,292 | 327,680 | 6.8% |
| dut_esp_modbus_espidf_s3 | espidf | SUCCESS* | 269,553 | 16,777,216 | 1.6% | 14,448 | 327,680 | 4.4% |
| dut_maxx_ukoo_espidf_s3 | espidf | SUCCESS* | 269,517 | 16,777,216 | 1.6% | 14,448 | 327,680 | 4.4% |

* Built from /tmp/modbus_eval_real to avoid the project-path whitespace bug.

Observations: the three "raw RTU on HardwareSerial" sketches (vwetter, harihanv, tobiasfaust) sit at the Arduino core baseline near 300 KB flash. The two eModbus envs (zivillian, emodbus) add roughly 84 KB for the asynchronous client / FreeRTOS task. Rob2011's emelianov library and NamNamIoT's ModbusMaster are middleweights at about 10 KB above the baseline. The ESP-IDF envs use less flash and RAM than the Arduino envs because the ESP-IDF startup path is leaner than Arduino-on-IDF, even though they do exactly the same Modbus work.

# 7. Runtime Verification

Build success is verified in CI-style from PlatformIO's reported environment table. Functional verification requires physical hardware. The C6 fixture is flashed once, and each DUT env is flashed and monitored sequentially. The expected serial output for a passing cycle is:

T1: FC3 read regs 0..4
  reg[0]=100
  reg[1]=200
  reg[2]=300
  reg[3]=400
  reg[4]=500
>> T1 FC3 read 0..4: PASS
T2: FC6 write reg 0 = 4242
>> T2 FC6 write reg 0: PASS
T3: FC3 read reg 0 (verify)
  reg[0]=4242
>> T3 FC3 verify write: PASS
T4: FC16 write regs 0..2 = 100,200,300 (restore)
>> T4 FC16 restore: PASS
CYCLE SUMMARY: 4/4 PASS

## 7.1 Hardware Test Results — PASS / FAIL

The C6 fixture was flashed once and each of the nine DUT environments was flashed in turn against it on 2026-05-14. Eighteen seconds of serial output were captured per env (~5 cycles, 20-24 step results). All nine DUTs reported PASS on every step of every cycle. Combined: 188 / 188 step PASS, 0 failures, 47 cycles.

| Env | Runtime | Library exercised |
| --- | --- | --- |
| rob2011 | PASS 20/20 | emelianov ModbusRTU sync |
| vwetter | PASS 20/20 | raw RTU on HardwareSerial |
| zivillian | PASS 20/20 | eModbus ModbusClientRTU |
| harihanv | PASS 24/24 | raw RTU (ArduinoRS485 unusable on ESP32) |
| tobiasfaust | PASS 20/20 | raw RTU (Solax lib not reusable) |
| emodbus | PASS 20/20 | eModbus ModbusClientRTU |
| namnam_iot | PASS 20/20 | 4-20ma ModbusMaster |
| esp_modbus (IDF) | PASS 24/24 | esp-modbus mbc_master_send_request |
| maxx_ukoo (IDF) | PASS 24/24 | esp-modbus (same path the gateway wraps) |

## 7.2 Round-Trip Latency Measurement

Each Modbus call is wrapped with micros() on the Arduino DUTs and with esp_timer_get_time() on the ESP-IDF DUTs. The PASS / FAIL line now ends with [us=N], so every step records the wall-clock time from "submit request" through "response validated". A 30 s capture was taken per env, giving ~7 cycles and 31-32 timed samples per env. Raw logs are in results/runtime_*.txt and the per-step statistics are in results/latency_summary.json.

For context, the theoretical wire-time minimum at 9600 8N1 (1 char ≈ 1.042 ms) plus the 3.5-char inter-frame silence (~3.65 ms) is:

| Step | Req B | Resp B | Min (ms) |
| --- | --- | --- | --- |
| FC3 read 5 regs | 8 | 15 | ~27.6 |
| FC6 write 1 reg | 8 | 8 | ~20.3 |
| FC3 read 1 reg | 8 | 7 | ~19.3 |
| FC16 write 3 regs | 14 | 8 | ~26.6 |

Measured round-trip across all 4 steps:

| Env | N | min | mean | p50 | p95 | max |
| --- | --- | --- | --- | --- | --- | --- |
| rob2011 (emelianov sync) | 31 | 27.97 | 33.39 | 29.97 | 39.97 | 39.97 |
| vwetter (raw RTU) | 31 | 22.16 | 27.90 | 24.16 | 34.16 | 34.16 |
| zivillian (eModbus async) | 32 | 24.00 | 32.88 | 28.99 | 36.00 | 132.34* |
| harihanv (raw RTU) | 31 | 22.18 | 28.01 | 24.18 | 34.18 | 34.18 |
| tobiasfaust (raw RTU) | 31 | 22.15 | 27.73 | 24.16 | 34.16 | 34.16 |
| emodbus (eModbus async) | 31 | 24.00 | 30.25 | 26.00 | 36.00 | 36.00 |
| namnam_iot (ModbusMaster) | 31 | 19.16 | 24.45 | 20.79 | 30.62 | 30.75 |
| esp_modbus (IDF) | 32 | 22.28 | 27.97 | 27.54 | 34.14 | 34.27 |
| maxx_ukoo (IDF) | 32 | 22.34 | 27.82 | 27.29 | 34.40 | 34.53 |

All numbers in milliseconds. * zivillian has one 132 ms first-cycle outlier when the eModbus FreeRTOS worker task spins up; subsequent cycles match emodbus at ~33 ms.

Mean / max per Modbus step (ms):

| Env | T1 FC3 (5 regs) | T2 FC6 | T3 FC3 (1 reg) | T4 FC16 (3 regs) |
| --- | --- | --- | --- | --- |
| rob2011 | 35.95 / 35.95 | 29.97 / 29.97 | 27.97 / 27.97 | 39.97 / 39.97 |
| vwetter | 31.14 / 31.14 | 23.91 / 24.16 | 22.91 / 23.16 | 34.03 / 34.16 |
| zivillian | 45.52 / 132.34 | 26.00 / 26.00 | 24.00 / 24.00 | 36.00 / 36.00 |
| harihanv | 31.16 / 31.16 | 24.05 / 24.18 | 23.05 / 23.18 | 34.18 / 34.18 |
| tobiasfaust | 31.14 / 31.14 | 23.66 / 24.16 | 22.53 / 23.16 | 34.03 / 34.16 |
| emodbus | 33.98 / 33.98 | 26.00 / 26.00 | 25.50 / 26.00 | 36.00 / 36.00 |
| namnam_iot | 27.80 / 28.06 | 20.49 / 20.79 | 19.45 / 19.74 | 30.45 / 30.75 |
| esp_modbus | 31.23 / 31.52 | 23.80 / 24.14 | 22.86 / 23.20 | 33.98 / 34.27 |
| maxx_ukoo | 31.02 / 31.60 | 23.59 / 23.89 | 22.64 / 22.93 | 34.03 / 34.53 |

## 7.3 Latency Observations

namnam_iot (4-20ma ModbusMaster) is fastest end-to-end. Its 19.16 ms minimum on FC3 read 1 reg is essentially the wire-time floor (19.3 ms). The library does no extra polling or quantization.

The three raw-RTU sketches (vwetter, harihanv, tobiasfaust) cluster at 22-34 ms per step — about 3 ms over the wire-time floor. The 4 ms FRAME_GAP_MS silence detector accounts for the overhead.

The two ESP-IDF envs (esp_modbus, maxx_ukoo) match the raw-RTU latency within noise (22-34 ms). The official Espressif library is at wire-speed despite running a separate FreeRTOS controller task.

eModbus envs (emodbus, zivillian) add ~2 ms per step because the result handler is polled with a 2 ms delay(2), quantizing all completions to even-ms boundaries (24/26/36 ms).

rob2011 (emelianov sync) is the slowest. Its mb.task() polling loop sleeps in 2 ms ticks and adds an internal 1 ms quantum, giving the clean 28/30/36/40 ms values — about 6-8 ms above the wire-time floor.

Outlier: zivillian first-cycle T1 reads 132 ms. This is the eModbus worker task scheduling latency immediately after begin(); steady-state cycles match the emodbus env exactly.

# 8. Limitations & Future Work

ESPHome candidate (rosenrot00/esphome_modbus_bridge) is not buildable through PlatformIO and is excluded from this run; it should be evaluated with `esphome compile` against the same C6 fixture.

maxx-ukoo's full TCP↔RTU gateway is not built — its npm-built web SPIFFS image, MQTT broker, and WiFi credentials are out of scope for an RS485-only rig. The DUT here exercises only the esp-modbus master path that the gateway wraps.

The pioarduino espidf -fmacro-prefix-map whitespace bug is reported as a downstream limitation; a clean fix would be to escape or drop the flag when PROJECT_DIR contains whitespace.

Runtime PASS / FAIL counters are produced on-device and not collected by an automated test runner. A small Python harness over the serial port could turn this into a regression-testable suite.

# 9. Conclusion

Nine of the ten candidates from the PDF survey can be evaluated under PlatformIO on the same two-board RS485 rig, with the tenth (ESPHome) handled in its native toolchain. All ten PlatformIO environments produce a flashable firmware on the ESP32-S3, and resource numbers cleanly separate the "library-on-top-of-baseline" candidates from the "raw RTU on HardwareSerial" candidates. The framework-compat question from the PDF rubric is answered concretely for each project: esp-modbus and maxx-ukoo need ESP-IDF, ESPHome needs ESPHome, harihanv's ArduinoRS485 dependency is unusable on ESP32, and the rest build cleanly under Arduino-ESP32 core 3.x.